🧠 PAX A920 RS232 Custom Cable Documentation (GHL UOB Integration)

✅ Confirmed Working Wiring

DB-9 Female (to USB RS232 Adapter) → RJ45 (to PAX Docking Station L920-BE)

| DB-9 Female Pin | Signal | T568B RJ45 Pin | T568B Color |
| --- | --- | --- | --- |
| Pin 1 | DCD | Pin 7 | White Brown |
| Pin 3 | TXD | Pin 5 | White Blue |
| Pin 4 | DTR | Pin 6 | Green |

📌 This pinout was confirmed working with GHL PAX A920 L920-BE Dock and COM port simulator.

🧷 RJ45 Layout (T568B Standard)

Clip Facing Away From You (Left to Right):

1. White Orange

2. Orange

3. White Green

4. Blue

5. White Blue  → TXD DB9

6. Green       → DTR DB9

7. White Brown → DCD DB9

8. Brown

🔌 DB9 RS232 Pinout Reference

| DB-9 Pin | Signal Name | Description |
| --- | --- | --- |
| 1 | DCD (Data Carrier Detect) | Used by dock communication |
| 2 | RXD (Receive Data) | (Not used in this setup) |
| 3 | TXD (Transmit Data) | Data sent from PC to POS |
| 4 | DTR (Data Terminal Ready) | Handshake Signal |
| 5 | GND (Ground) | Unused in their official cable |
| 6 | DSR (Data Set Ready) | (Not used in this setup) |
| 7 | RTS (Request To Send) | (Not used in this setup) |
| 8 | CTS (Clear To Send) | (Not used in this setup) |
| 9 | Ring Indicator (RI) | (Not used in this setup) |

🧰 Required Components

- PL2303GT USB to Serial Adapter

  - ⚠️ Must be Prolific chipset (PL2303 preferred)

  - FTDI chipset didn’t work at all during tests

- RS232 DB9 Female connector

- T568B standard network cable (Cat5e/Cat6)

- Crimping tool + RJ45 connector

⚙️ Serial Port Settings (from POS Integration Spec Appendix C)

| Setting | Value |
| --- | --- |
| Baud Rate | 9600 |
| Data Bits | 8 |
| Parity | None |
| Stop Bits | 1 |
| Flow Control | None |
| Protocol | Character Mode (P_char_mode) |

Set this in Device Manager > COM Port > Properties > Port Settings.

🧪 Testing Tips

- You can send HEX commands using:

  - Python (via pyserial)

  - TeraTerm or PuTTY (Hex mode, Serial COM)

- Use a POS simulator tool or a prebuilt .exe from GHL to simulate SALE/VOID (SET IT TO COM1)

- Confirm terminal responds with:

- Error Code: 00

- Approval: <some code>

📸 Reference Photo

Wiring Diagram